Rapport ISC annuel

Année 2020

Annual ISC report

Year 2020

L’ensemble des informations publiées dans le présent rapport ont volontairement été anonymisées pour des raisons de respect du droit à la vie privée des proriétaires/conducteurs des véhicules testés.

Abstract

L’ensemble des tests ISC réalisés, pour l’année 2020, par le constructeur (18) et par l’autorité wallonne de réception par type (6) ont abouti à des résultats conformes à la règlementation en vigueur.

L’autorité wallonne a été confrontée à quelques difficultés pour la réalisation de ces tests :

- Comment obtenir la liste des véhicules immatriculés qui sont potentiellement à soumettre aux essais ISC ?

- Comment inciter/obliger les propriétaires de véhicules à donner leur autorisation de soumettre leur véhicule aux essais ISC ?

La réponse à ces interrogations pourraient être apportée par l’instauration de bases légales permettant :

- de collecter aisément les informations nécessaires (c’est-à-dire tant les informations techniques relatives aux véhicules que les coordonnées de contact des propriétaires de ceux-ci) ou la mise en œuvre d’une base de données européenne des véhicules immatriculés en Europe.

- d’obliger ou, au minimum, d'inciter fortement les propriétaires de véhicules à mettre ceux-ci  à disposition de l'autorité pour la réalisation des essais imposés par la règlementation relative à la conformité en service

All the ISC tests realised for 2020 by the manufacturer (18) and by the Walloon type approval authority (6) comply with the regulation.

The Walloon authority has faced some difficulties in carrying out these tests:

- How to obtain the list of registered vehicles that are potentially to be submitted to ISC tests ?

- How to encourage/require from vehicle owners to give their authorization to submit their vehicle to ISC testing ?

The answer to these questions could be provided by the establishment of legal bases allowing:

- to easily collect the necessary information (i.e. both technical information relating to vehicles and contact details of their owners) or the implementation of a European database of vehicles registered in Europe.

- to force legally or, at least, to strongly encourage vehicle owners to make their vehicle available to the authority for carrying out the tests required by the regulations relating to in-service conformity.

Table des matières

Bref aperçu et conclusions principales	4

Quick overview and main conclusions	5

Activités ISC menées par le constructeur au cours de l’année précédente	6

Collecte des informations par le constructeur	6

Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais).	6

Activités ISC menées par les laboratoires accrédités ou les services techniques au cours de l’année précédente	13

Collecte des informations et évaluation des risques	13

Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais)	13

Activités ISC menées par l’autorités responsable de l’octroi de la réception par type	14

Collecte des informations et évaluation des risques	14

Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais)	15

Enquêtes approfondies	18

Mesures correctives	18

Evaluation de la diminution annuelle escomptée des émissions résultant des mesures correctives ISC	19

Enseignements tirés (y compris en matière de performance des instruments utilisés	20

Questionnaire d’interview des propriétaires/conducteurs habituels des véhicules en vue de vérifier le respect des critères de sélection	20

Installation du PEMS sur des véhicules non équipés d’attelage	20

Base de données européennes en matière d’immatriculation des véhicules	20

Base règlementaire contraignante ou fortement incitative à la mise à disposition des véhicules par les propriétaires/ conducteurs habituels	21

Rapport concernant d’autres essais non valides	22

Annexes	23

Documents relatifs aux essais ISC 2020 du constructeur	23

ANNEXE I - Summary tests reports (18) ;	23

ANNEXE II - Certificate of conformity (18) ;	41

ANNEXE III - Rapport annuel du constructeur ;	62

Documents relatifs aux essais ISC 2020 de l’autorité	68

ANNEXE IV – Rapport ISC 2020 de l’ISSeP ;	68

ANNEXE V - Rapport ISC 2020 d’Horiba ;	69

ANNEXE VI - Certificat de conformité des véhicules testés (6) ;	70

ANNEXE VII - Tableaux « critères de sélection » (reprenant l’interview du propriétaire et l’inspection du véhicule).	78

# Bref aperçu et conclusions principales

(English below)

Ce rapport est publié conformément aux exigences formulées au sein du règlement (UE) 2017/1151 - du 1 er juin 2017 - complétant le règlement (CE) n°715/2007 du Parlement européen et du Conseil relatif à la réception des véhicules à moteur au regard des émissions des véhicules particuliers et utilitaires légers (Euro 5 et Euro 6) et aux informations sur la réparation et l’entretien des véhicules, modifiant la directive 2007/46/CE du Parlement européen et du Conseil, le règlement (CE) n°692/2008 de la Commission et le règlement (UE) n°1230/2012 de la Commission et abrogeant le règlement (CE) n°692/2008 – tel que modifié. Il répond plus particulièrement au point 8 de la partie B (Nouvelle méthodologie applicable à la conformité en service) de l’annexe II (Conformité en service) qui stipule :

« Rapport annuel de l'autorité responsable de l'octroi de la réception par type :

L'autorité responsable de l'octroi de la réception par type met à disposition gratuitement, sur un site web publiquement accessible et sans que l'utilisateur ne soit contraint de révéler son identité ou de s'enregistrer, un rapport contenant les résultats de toutes les enquêtes ISC menées à terme au cours de l'année précédente, au plus tard le 31 mars de chaque année. Si certaines enquêtes ISC de l'année précédente sont encore ouvertes à cette date, elles doivent être publiées dès que l'enquête est clôturée. Le rapport doit contenir au minimum les éléments énoncés à l'appendice 4 ».

Il est établi de façon à répondre aux exigences de présentation formulées à l’appendice 4 ci-dessus évoquée.

Il reprend les principales informations et conclusions des tests et essais ISC réalisés au cours de l’année précédente – tant par le constructeur que par l’autorité ou encore par les laboratoires accrédités ou services techniques (si applicable).

ESSAI ISC 2020 – CONSTRUCTEUR

Selon 1) la date de la première émission du Certificat de Conformité, 2) le volume des ventes total pour chaque famille (tests ISC requis uniquement si les ventes totales de l’année précédente de la famille concernée excède 5000 unités) et 3) la fréquence minimale des essais que requière la règlementation (le délai entre deux vérifications de conformité en service pour une famille ISC ne peut être supérieur à 24 mois), le nombre de familles à tester par le constructeur en 2020 s’élève à 6.

Le nombre de véhicules à tester au sein de chaque famille dépend du volume d’immatriculations européennes escomptées (< 100.000 è 1 lot d’échantillons à tester ; > 100.001 mais < 200.000 è 2 lots à tester ; > 200.001 è 3 lots à tester). Pour 2020, le nombre de lots pour chaque famille est établit à 1.

La taille minimale d’un échantillon pour un résultat d’acceptation est de 3 véhicules et la taille cumulée d’un échantillon est de maximum 10 véhicules pour les essais du type 1. Le constructeur a donc testé 18 véhicules.

Les véhicules sélectionnés (soit 3 véhicules par famille) répondant aux limites d’émission établies par la règlementation 715/2007, une décision d’acceptation de l’échantillon a pu être prise (résultat « PASS » de l’arbre décisionnel) pour les six familles sur base de ces 18 véhicules.

ESSAI ISC 2020 – AUTORITE

L’autorité wallonne a délivré des « réceptions CE par type d’un véhicule en ce qui concerne les émissions » à un seul constructeur. Ce constructeur, au moment de la sélection des véhicules, disposait de 14 familles ISC différentes. Dès lors, étant entendu que la règlementation impose que 5% au moins de famille ISC par constructeur et par an ou au moins deux familles ISC par constructeur et par an soient soumises aux essais, 2 familles ISC ont été testées. Cela représente un échantillon de 6 véhicules conformément au tableau décisionnel applicable pour la procédure statistique. En effet, les véhicules sélectionnés (soit 3 véhicules par famille) répondant aux limites d’émission établies par la règlementation 715/2007, une décision d’acceptation de l’échantillon a pu être prise (résultat « PASS » de l’arbre décisionnel) pour les deux familles sur base de ces 6 véhicules.

# Quick overview and main conclusions

(French above)

This report is published according to regulation (UE) 2017/1151.

“Annual report by the granting type approval authority

The granting type approval authority shall make available on a publicly accessible website, free of charge and without the need for the user to reveal their identity or sign up, a report with the results of all the finalised ISC investigations of the previous year, at the latest by the 31 March of each year. In case some ISC investigations of the previous year are still open by that date, they shall be reported as soon as the investigation is finalised. The report shall contain at least the items listed in Appendix 4.”

It contains information and conclusions about ISC tests conducted in 2020 by both manufacturer and authority.

ISC TESTS 2020 – MANUFACTURER

According to 1) the date of the first Certificate of Conformity issued, 2) the total sales for each family (ISC tests required only if the total sales for the family in the past year exceed 5000) and 3) the minimal frequency (the time period between commencing two in-service conformity checks by the manufacturer for an ISC family shall not exceed 24 months), the manufacturer had to test 6 ISC families in 2020.

The number of vehicles to be tested depends on the volume of European registrations (< 100.000 è 1 sample lot to be tested; > 100.001 but < 200.000 è 2 sample lots; > 200.001 è 3 sample lots). In 2020, the number of lots for each family is fixed at 1.

The minimum sample size for a pass result is three vehicles, and the maximum cumulative sample size is ten vehicles for the Type 1. So, the manufacturer had test 18 vehicles.

All selected vehicles (i.e. 3 vehicles per family) complying with the emission limits established by regulation 715/2007, a « sample pass » outcome has been reached for six selected families based on these 18 vehicles.

ISC TESTS 2020 – AUTHORITY

The Walloon authority delivered « EC type-approvals of a vehicle with regard to emissions » to one manufacturer. This manufacturer has 14 different ISC families. The regulation requires that a minimum of 5 % of the ISC families per manufacturer per year or at least two ISC families per manufacturer per year must be tested. So, 2 ISC families have been tested. It represents a sample of 6 vehicles in accordance to the applicable decision chart for a statistical procedure. Indeed, all selected vehicles (i.e. 3 vehicles per family) complying with the emission limits established by regulation 715/2007, a « sample pass » outcome has been reached for both selected families based on these 6 vehicles.

# Activités ISC menées par le constructeur au cours de l’année précédente

## Collecte des informations par le constructeur

La division DTS (en Allemagne) de HME dispose d’une liste des véhicules de société en circulation sur le territoire allemand.

Dès que les modèles de voitures à tester (selon la famille ISC) sont connus, ils consultent celle-ci pour trouver des véhicules qui pourraient potentiellement être sélectionnés pour la réalisation des essais.

En parallèle, la division HCC (un centre d'appels du constructeur) contacte les clients de la firme (propriétaires des véhicules) pour leur demander s'ils sont intéressés de participer aux essais ISC des familles sélectionnées. Des incitants sont proposés aux clients en vue de favoriser leur acceptation à soumettre leur véhicule aux essais ISC (belle voiture de remplacement et un entretien gratuit).

## Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais).

PLANIFICATION

Ci-après le planning de tests établi par le constructeur pour la réalisation de ses essais ISC :

Pour 2020, 6 familles ISC ont donc été testées par le constructeur (essais de type 1).

SELECTION

Les véhicules soumis aux essais sont issus de la flotte de véhicules de l’entreprise et/ou de clients allemands.

Extrait du rapport du constructeur :

« Procured from company car fleet and customers »

Extrait du rapport du constructeur :

« According to the list in Appendix 1 of Annex II of EU 2017/1151 »

Les véhicules ont été sélectionnés conformément à la liste de critères établie à l’appendice 1 de l’annexe II du règlement européen 2017/1151.

Les résultats de la vérification, par le constructeur, de la conformité des véhicules aux critères de sélection sont disponibles en annexe du présent rapport.

RESULTATS FINAUX des essais WLTP (type 1)

Famille 6-JHM-08-0 – HR-V/CR-V 1.5

La famille est composée de véhicules construits sur la période 2018/2019.

Les informations ont été collectées par le constructeur en août 2020.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d-Temp.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

Famille 6-JHM-13-0 – JAZZ 1.3

La famille est composée de véhicules construits sur la période 2018/2019.

Les informations ont été collectées par le constructeur en août 2020.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d-Temp.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

Famille 6-JHM-16-0 – CR-V-Hybrid

La famille est composée de véhicules construits sur la période 2018/2019.

Les informations ont été collectées par le constructeur en août 2020.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d-Temp.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

Famille 6-JHM-18-0 – Civic 1.6

La famille est composée de véhicules construits sur la période juin2018/septembre2018.

Les informations ont été collectées par le constructeur sur la période allant d’avril 2020 à octobre 2020.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

Famille 6-JHM-19-0 – Civic 1.0

La famille est composée de véhicules construits sur la période juin2018/septembre2018.

Les informations ont été collectées par le constructeur sur la période allant d’août 2019 à décembre 2019.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

Famille 6-JHM-20-0 – Civic 1.5

La famille est composée de véhicules construits sur la période 2018/2019.

Les informations ont été collectées par le constructeur sur la période allant d’avril 2020 à octobre 2020.

La valeur limite applicable pour les émissions de type 1 est la norme Euro 6d.

Aucune modification et / ou pièce suspectes dans l'historique de maintenance n’ont été relevées.

L'huile moteur et le filtre à air ont été remplacés avant le test d'émission.

Trois véhicules ont été soumis aux tests d'émissions sans aucun rejet.

CONCLUSION

L’ensemble des 18 véhicules testés (3 véhicules par familles pour les 6 familles testées en 2020) est conforme aux normes imposées par la règlementation.

# Activités ISC menées par les laboratoires accrédités ou les services techniques au cours de l’année précédente

## Collecte des informations et évaluation des risques

Non applicable. Aucun test n’a été réalisé par un laboratoire accrédité ou en service technique en 2020.

## Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais)

Non applicable. Aucun test n’a été réalisé par un laboratoire accrédité ou en service technique en 2020.

# Activités ISC menées par l’autorités responsable de l’octroi de la réception par type

Afin de garantir l’entière impartialité des laboratoires réalisant les essais de conformité en service et ainsi rencontrer les recommandations formulées par le Conseil européen, le Gouvernement wallon a décidé, en 2019, de se doter d’un laboratoire wallon permettant la réalisation des essais ISC rendus obligatoires par l’application du règlement européen 2017/1151.

Le Gouvernement wallon a, alors, confié à l’ISSeP la mission d’élaborer et de faire construire ce laboratoire de tests en ses infrastructures de Colfontaine. L’ISSeP a, par la même occasion, été désigné comme laboratoire de référence pour la réalisation des essais ISC de l’autorité régionale wallonne.

En raison de la situation sanitaire, la construction du laboratoire a été retardée et les essais 2020 n’ont pu être réalisés au sein des infrastructures de l’ISSeP.

Néanmoins, afin de rencontrer les obligations imposées à l’autorité régionale wallonne consécutivement à l’entrée en vigueur de la règlementation européenne relative à la conformité en service des véhicules, l’ISSeP a, à la suite d’un marché public, fait réaliser, sous sa responsabilité, les essais ISC 2020 par la société Horiba – Oberursel – Allemagne (ISO 17025 et ISO 17020).

## Collecte des informations et évaluation des risques

COLLECTE DES INFORMATIONS

L’autorité régionale wallonne a rencontré des difficultés à obtenir une liste des véhicules (relevant de l’obligation de tests de conformité en service) en circulation sur le territoire national.

EVALUATION DU RISQUE

L’analyse du risque pour la sélection des deux familles à tester en 2020 a essentiellement porté sur les quotas de vente des différentes familles relevant de l’autorité wallonne. Celles dont les volumes de ventes étant les plus importants ont dès lors été privilégiées.

Légendes : en surligné jaune : les familles sélectionnées par l’autorité ; en police d’écriture rouge : les familles sélectionnées par le constructeur

Il a été décidé de sélectionner :

- Une famille faisant également l’objet en 2020 d’essais ISC par le constructeur afin de pouvoir confronter les résultats obtenus pas les différentes parties. Il s’agit de la famille 6-JMH-08-0 ;

- Une famille non testée en 2020 par le constructeur et dont le volume de vente annuelle 2019 était conséquent. Il s’agit de la famille 6-JMH-14-0.

## Contrôles ISC (comprenant la planification et la sélection des familles soumises aux essais, ainsi que les résultats finaux des essais)

PLANIFICATION

Le nombre de familles relevant de l’autorité wallonne dans le cadre des essais ISC conduit à la sélection et au testing de deux familles ISC par constructeur par an.

En effet, ce nombre de famille s’élevant actuellement à 14, le nombre minimal de 2 familles par an par constructeur est d’application étant entendu qu’il est plus important que le quota de 5% des familles par an par constructeur.

Les moteurs à propulsion diesel étant peu représentatifs en termes de volume de vente pour ce qui concerne ce constructeur, l’accent n’est pas mis sur ce type de motorisation.

SELECTION

La sélection des familles s’est faite selon la méthodologie explicitée supra, au point  Évaluation des risques .

Les véhicules soumis aux essais ISC 2020 sont 6 véhicules immatriculés en Belgique.

Les véhicules sélectionnés répondent aux critères de sélection tels que définit à l’appendice 1 de l’annexe II du règlement européen 2017/1151.

Les résultats de la vérification de la conformité des véhicules aux critères de sélection sont disponibles en annexe du présent rapport.

RESULTATS FINAUX

Famille 6-JHM-08-0 – CR-V

Essai WLTP sur banc à rouleaux (type 1)

Les émissions lors de l’essai WLTP des 3 véhicules respectent les limites imposées par la réglementation. Les résultats obtenus pour chacun des 3 véhicules sont relativement similaires. Les émissions de CO2 combinées ainsi que la consommation de carburant vont de légèrement inférieures à pratiquement identiques à celles annoncées sur le certificat de conformité du véhicule.

Toutes les vérifications et manipulations préalables à la réalisation des essais ont été effectuées conformément à la règlementation ISC. Il appert que les véhicules sont en bonne condition de fonctionnement avec tous leurs composants non-modifiés et non-endommagés.

Les essais sur banc ont été réalisés en désactivant le système de sécurité électronique du véhicule.

Essai Real Driving Emissions (type 1a)

Les émissions de CO, de NOx et de particules présentés sont les émissions finales calculées suivant la règlementation RDE. Les émissions de CO2 urbaine/totale présentées sont calculées en divisant la masse de CO2 urbaine/total émises par la distance respective urbaine/totale parcourue.

Les émissions lors de l’essai RDE des 3 véhicules repris dans les tableaux ci-dessus respectent les limites imposées par la réglementation.

Famille 6-JHM-14-0 – HR-V

Essai WLTP sur banc à rouleaux (type 1)

Les émissions lors de l’essai WLTP des 3 véhicules respectent les limites imposées par la réglementation. Les résultats obtenus pour chacun des 3 véhicules sont relativement similaires. Les émissions de CO2 combinées ainsi que la consommation de carburant vont de légèrement inférieures à pratiquement identiques à celles annoncées sur le certificat de conformité du véhicule.

Toutes les vérifications et manipulations préalables à la réalisation des essais ont été effectuées conformément à la règlementation ISC. Il appert que les véhicules sont en bonne condition de fonctionnement avec tous leurs composants non-modifiés et non-endommagés.

Les essais sur banc ont été réalisés en désactivant le système de sécurité électronique.

Essai Real Driving Emissions (type 1a)

Les émissions de CO, de NOx et de particules présentés sont les émissions finales calculées suivant la règlementation RDE. Les émissions de CO2 urbaine/totale présentées sont calculées en divisant la masse de CO2 urbaine/total émises par la distance respective urbaine/totale parcourue.

Les émissions lors de l’essai RDE des 3 véhicules repris dans les tableaux ci-dessus respectent les limites imposées par la réglementation.

CONCLUSION

L’ensemble des 6 véhicules testés (3 véhicules par familles pour les 2 familles en 2020) sont conformes aux normes imposées par la règlementation.

## Enquêtes approfondies

Non applicable au vu des résultats d’essais conformes à la règlementation en vigueur.

## Mesures correctives

Non applicable au vu des résultats d’essais conformes à la règlementation en vigueur.

# Evaluation de la diminution annuelle escomptée des émissions résultant des mesures correctives ISC

Non applicable. Aucune mesure corrective n’a dû être prise au vu des résultats d’essais conformes à la règlementation en vigueur.

# Enseignements tirés (y compris en matière de performance des instruments utilisés

## Questionnaire d’interview des propriétaires/conducteurs habituels des véhicules en vue de vérifier le respect des critères de sélection

Il importe de documenter au maximum les réponses apportées par les propriétaires/conducteurs habituels des véhicules lors du questionnaire visant à vérifier le respect du véhicule aux critères de sélection imposés par l’appendice 1 de l’annexe II du règlement européen 2017/1151.

Cette documentation peut aisément être apportée par le biais de photographies.

Cet enseignement découle de deux constats rencontrés entre la réalisation de l’interview du propriétaire d’une part et l’inspection concrète du véhicule par le laboratoire d’autre part :

- Le kilométrage du véhicule annoncé par le propriétaire lors de l’interview différait du kilométrage relevé au compteur lors de l’inspection du véhicule. Ceci a eu pour conséquence que le véhicule a dû être retiré de l’échantillonnage de test et remplacé par un autre véhicule (de la réserve de sélection) afin de répondre aux exigences règlementaires ;

- Le type de monte de pneus annoncé par le propriétaire lors de l’interview différait du type de monte de pneus constaté lors de l’inspection du véhicule. Ceci a eu pour conséquence que la monte pneumatique a dû être changée avant la réalisation des essais afin de correspondre aux prescriptions du certificat de conformité.

Par ailleurs, un contact avec les propriétaires des véhicules est rendu indispensable par le biais de ce questionnaire. Cette obligation rend l’accès difficile aux véhicules immatriculés dans des pays où la langue officielle est différente de celle du pays de l’autorité dont relève la charge de réaliser les essais ISC.

## Installation du PEMS sur des véhicules non équipés d’attelage

L’installation et plus particulièrement le raccordement du système de mesure des émissions PEMS sur les véhicules non équipés d’attelage (attache-remorque) ne sont pas aisées.

Des alternatives de greffage de l’appareillage sont alors imaginées et mises en œuvre en fonction du bon vouloir créatif du laboratoire qui réalise les essais.

Ces dispositifs peuvent, cependant, induire la survenue de dommages sur le véhicule.

Ce constat amène à être extrêmement attentifs à la documentation du dossier à la réception et au départ des véhicules.

## Base de données européennes en matière d’immatriculation des véhicules

Les difficultés (voire même l’impossibilité) rencontrées par l’autorité wallonne de réception par type (en charge de la réalisation des essais ISC) à disposer d’une liste de véhicules (répondant à des critères bien spécifiques) en circulation sur le territoire national et/ou européen amènent à plaider soit pour la mise en œuvre d’une base de données européenne des véhicules immatriculés en Europe soit pour l’instauration d’une base juridique permettant de collecter aisément les informations nécessaires (c’est-à-dire tant les informations techniques relatives aux véhicules que les coordonnées de contact des propriétaires de ceux-ci).

## Base règlementaire contraignante ou fortement incitative à la mise à disposition des véhicules par les propriétaires/ conducteurs habituels

L’autorité de réception par type est soumise à l’obligation de réaliser annuellement des essais ISC sur des véhicules en service. Ces véhicules sont donc la propriété, voire un outil de travail et/ou, à tout le moins, dans la plupart des cas un moyen de transport d’utilité quotidienne des citoyens européens.

L’autorité de réception par type est dès lors très fréquemment confrontée à de nombreux refus de ces propriétaires/conducteurs habituels des véhicules qui sont peu enclin à se passer de leur véhicule le temps de la réalisation des essais (soit en moyenne 5 à 7 jours ouvrables).

Ce constat amène à se questionner sur l’instauration d’une base règlementaire contraignante ou, au minimum, fortement incitative à la mise à disposition des véhicules aux essais imposés par la règlementation relative à la conformité en service.

# Rapport concernant d’autres essais non valides

Non applicable. Aucun essai non valide n’est à rapporter.

# Annexes

## Documents relatifs aux essais ISC 2020 du constructeur

### ANNEXE I - Summary tests reports (18) ;

### ANNEXE II - Certificate of conformity (18) ;

### ANNEXE III - Rapport annuel du constructeur ;

Double cliquer sur les images ci-dessous pour accéder aux documents complets

## Documents relatifs aux essais ISC 2020 de l’autorité

### ANNEXE IV – Rapport ISC 2020 de l’ISSeP ;

Double cliquer sur l’image ci-dessous pour accéder au document complet

### ANNEXE V - Rapport ISC 2020 d’Horiba ;

Double cliquer sur l’image ci-dessous pour accéder au document complet

### ANNEXE VI - Certificat de conformité des véhicules testés (6) ;

ANNEXE VII - Tableaux « critères de sélection » (reprenant l’interview du propriétaire et l’inspection du véhicule).

6-JHM-08-0 – Vehicle n°1 - CR-V - 209302

| Registration plate number | XXX |
| --- | --- |
| Mileage | 43430 km |
| Date of first registration | 25.02.2019 |
| VIN | JHMRW2740KX209302 |
| Emission class and character | Euro 6 / AG |
| Country of registration (in EU only) | Belgique |
| Model | Honda CR-V (RW/RW201/1) |
| Engine code | L15BY |
| Engine volume (l) | 1498 |
| Engine power (kW) | 127 |
| Gearbox type | Manual |
| Drive axle (FWD/AWD/RWD) | AWD |
| Tyre size (front and rear if different) | 235/60 R18 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs? | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 35 |
| % rural | 25 |
| % urban | 40 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | half |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No leaks |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leaks |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Okay |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | Safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Yes |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

6-JHM-08-0 – Vehicle n°2 - CR-V - 205599

| Registration plate number | XXX |
| --- | --- |
| Mileage | 38 872 km |
| Date of first registration | 28.08.2019 |
| VIN | JHMRW1740KX205599 |
| Emission class and character | Euro 6 DG |
| Country of registration (in EU only) | Belgique |
| Model | Honda CR-V (RW/RW101/1) |
| Engine code | L15BY |
| Engine volume (l) | 1.498 |
| Engine power (kW) | 127 |
| Gearbox type | Manual |
| Drive axle (FWD/AWD/RWD) | FWD |
| Tyre size (front and rear if different) | 235/60 R18 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs?. | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 75 |
| % rural | 10 |
| % urban | 15 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | Half |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No damage |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leaks |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Okay |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Yes |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

6-JHM-08-0 – Vehicle n°3 - CR-V - 211968

| Registration plate number | XXX |
| --- | --- |
| Mileage | 29 407 km |
| Date of first registration | 30.04.2019 |
| VIN | JHMRW2780KX211968 |
| Emission class and character | Euro 6 DG |
| Country of registration (in EU only) | Belgique |
| Model | Honda CR-V (RW/RW201/1) |
| Engine code | L15BY |
| Engine volume (l) | 1.498 |
| Engine power (kW) | 127 |
| Gearbox type | Manual |
| Drive axle (FWD/AWD/RWD) | AWD |
| Tyre size (front and rear if different) | 235/55 R19 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs? | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 20 |
| % rural | 10 |
| % urban | 70 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | Half |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No damage |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leaks |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Okay |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | Safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Yes |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

6-JHM-14-0 – Vehicle n°1 - HR-V - 211948

| Registration plate number | XXX |
| --- | --- |
| Mileage | 35 088 km |
| Date of first registration | 08.04.2019 |
| VIN | JHMRU1790KX211948 |
| Emission class and character | Euro 6 / AG |
| Country of registration (in EU only) | Belgique |
| Model | Honda HR-V (RU/RU102/1) |
| Engine code | L15BY |
| Engine volume (l) | 1.498 |
| Engine power (kW) | 96 |
| Gearbox type | Manual 6-Gear |
| Drive axle (FWD/AWD/RWD) | FWD |
| Tyre size (front and rear if different) | 215/55 R17 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs? | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 85 |
| % rural | 15 |
| % urban | 0 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | Quarter |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No damage |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leak |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Ok |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | Safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Before |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

6-JHM-14-0 – Vehicle n°2 - HR-V - 213231

| Registration plate number | XXX |
| --- | --- |
| Mileage | 36 865 km |
| Date of first registration | 28.05.2019 |
| VIN | JHMRU1790KX213231 |
| Emission class and character | Euro 6 / AG |
| Country of registration (in EU only) | Belgique |
| Model | Honda HR-V (RU/RU102/1) |
| Engine code | L15BY |
| Engine volume (l) | 1.498 l |
| Engine power (kW) | 96 kW |
| Gearbox type | Manual 6-Gear |
| Drive axle (FWD/AWD/RWD) | FWD |
| Tyre size (front and rear if different) | 215/55 R17 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs? | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 40 |
| % rural | 30 |
| % urban | 30 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | quarter |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No damage |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leaks |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Yes |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | Safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Yes |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

6-JHM-14-0 – Vehicle n°3 - HR-V - 219014

| Registration plate number | XXX |
| --- | --- |
| Mileage | 35 795 km |
| Date of first registration | 25.06.2019 |
| VIN | JHMRU1790KX219014 |
| Emission class and character | Euro 6 AG |
| Country of registration (in EU only) | Belgique |
| Model | Honda HR-V (RU/RU104/3) |
| Engine code | L15BY |
| Engine volume (l) | 1.498 |
| Engine power (kW) | 96 |
| Gearbox type | Manual |
| Drive axle (FWD/AWD/RWD) | FWD |
| Tyre size (front and rear if different) | 225/45 R18 |
| Is the vehicle involved in a recall or service campaign? If yes: Which one? Has the campaign repairs already been done? | No |

| Name of the owner | XXX |
| --- | --- |
| Contact | XXX |
| How many owners did the vehicle have? | 1 |
| Did the odometer not work? | No |
| Was the vehicle used for one of the following?: |  |
| As car used in showrooms? | No |
| As a taxi | No |
| As delivery vehicle | No |
| For racing/motorsports? | No |
| As a rental car? | No |
| Has the vehicle carried heavy loads over the specifications of the manufacturer? | No |
| Have there been major engine or vehicle repairs? | No |
| Have there been unauthorised major engine or vehicle repairs? | No |
| Has there been a power increase/tuning? | No |
| Was any part of the emissions after-treatment and/or the fuel system replaced? Were original parts used? | No |
| Was any part of the emissions after-treatment system permanently removed? | No |
| Were there any unauthorised devices installed (Urea killer, emulator, etc)? | No |
| Was the vehicle involved in a serious accident? Provide a list of damage and repairs done afterwards | No |
| Has the car been used with a wrong fuel type (i.e. gasoline instead of diesel) in the past? Has the car been used with non-commercially available EU-quality fuel (black market, or blended fuel?) | No |
| Where do you use your vehicle more often? |  |
| % motorway | 45 |
| % rural | 25 |
| % urban | 30 |
| In which country was the vehicle refuelled during the last two times? | EU |
| Has a fuel additive, not approved by the manufacturer been used? | No |
| Has the vehicle been maintained and used in accordance with the manufacturer's instructions? | Yes |
| Full service and repair history including any re-works | Yes |

| Fuel tank level (full / empty) Is the fuel reserve light ON? If yes, refuel before test. | empty |
| --- | --- |
| Are there any warning lights on the instrument panel activated indicating a vehicle or exhaust after-treatment system malfunctioning that cannot be resolve by normal maintenance? (Malfunction Indication Light, Engine Service Light, etc?) | No |
| Is the SCR light on after engine-on? If yes, the AdBlue should be filled in, or the repair executed before the vehicle is used for testing. | n/a |
| Visual inspection exhaust system Check leaks between exhaust manifold and end of tailpipe. Check and document (with photos). | No leaks |
| Exhaust gas relevant components Check and document (with photos) all emissions relevant components for damage. | No damage |
| Evap system Pressurize fuel-system (from canister side), testing for leaks in a constant ambient temperature environment, FID sniff test around and in the vehicle. | No leaks |
| Fuel sample Collect fuel sample from the fuel tank. | Yes |
| Air filter and oil filter Check for contamination and damage and change if damaged or heavily contaminated or less than 800 km before the next recommended change. | No damage |
| Window washer fluid (only for evap testing) Remove window washer fluid and fill tank with hot water. | n/a |
| Wheels front & rear Check whether the wheels are freely moveable or blocked by the brake. | Freely moveable |
| Tyres (only for evap testing) Remove spare tyre, change to stabilised tyres if the tyres were changes less than 15 000 km ago. Use summer and all season tyres only. | n/a |
| Drive belts & cooler cover Document with photos | No damage |
| Check fluid levels Check the max. and min. levels (engine oil, cooling liquid) / top up if below minimum | Okay |
| Filler flap (only for evap testing) Check overfill line within filler flap is completely free of residues or flush the hose with hot water. | n/a |
| Vacuum hoses and electrical wiring Check all for integrity. Document with photos | No damage |
| Injection valves/cabling Check all cables and fuel lines. Document with photos | No damage |
| Ignition cable (gasoline) Check spark plugs, cables, etc. In case of damage, replace them. | No damage |
| EGR & Catalyst, Particle Filter Check all cables, wires and sensors. Document with photos | No damage |
| Safety condition Check tyres, vehicle's body, electrical and braking system status are in safe conditions for the test and respect road traffic rules. | Safe |
| Semi-trailer Are there electric cables for semi-trailer connection, where required? | n/a |
| Aerodynamic modifications Verify no aftermarket aerodynamics modification that cannot be removed before testing was made (roof boxes, load racking, spoilers, etc.) and no standard aerodynamics components are missing (front deflectors, diffusers, splitters, etc.). Document with photos. | No modification |
| Check if less than 800 km away from next scheduled service, if yes, then perform the service | No |
| All checks requiring OBD connections to be performed before and/or after the end of testing | Yes |
| Powertrain Control Module calibration part number and checksum | Yes |
| OBD diagnosis (before or after the emissions test) Read Diagnostic Trouble Codes & Print error log | Yes |
| OBD Service Mode 09 Query (before or after the emissions test) Read Service Mode 09. Record the information. | Yes |
| OBD mode 7 (before or after the emissions test) Read Service Mode 07. Record the information | Yes |

ISSeP - Institut Scientifique de Service Public : https://www.issep.be/